ARMBOT

Starter Kit and Engineering Guide

by

Marvin Green

821 SW 14th, Troutdale, OR 97060

marvin@agora.rdrop.com

(503) 666-5907

Version 1.4

Introduction:

The ARMBOT is a simple 3 axis robotic arm, that is designed to use small R/C servos.  The ARMBOT provides clockwise and counter clockwise rotation of greater than 180 degrees, shoulder lift of greater than 45 degrees, an gripper range of about two inches.  The ARMBOT is designed to use the BOTBoard as the microcontroller, but can also use other controllers like a PIC or BASIC STAMP.

This kit is designed to spark your intuitive engineering skills.  It comes with assembly instructions, diagrams, parts list, and all the custom parts needed to build the kit.  This kit requires you to think, and be creative.  You should have some basic construction skills.  Please read the manual thoroughly before beginning, take your time and have fun.

The ARMBOT is simple in design, easy to build, and fun to use. I hope is that you have good time building and using it.

Marvin Green	7/31/2026

Parts provided with ARMBOT:

Base platform

Forearm platform -

Shoulder block -

Push rod tube - cut to size

Finger blocks - cut to make two fingers

Plastic shoulder hinge -

Parts you need to purchase.

2 - Royal Titan mini servo’s for forearm.

1 - Royal Titan standard servo for base mount.

1 - Package of 2, Aileron bellcrank for fingers.

1 - Package of 3, mini snap links

2 - Gear clamp.

4 - Rubber feet for base.

2 - Small pieces of foam rubber from finger grips.  [ The rubber feet for the base also 		     work well for the grippers. ]

1 - Push rod.  Sullivan .056 steel push rod cable part number SU-508.

1 - 2-56 Solid metal push rod with threaded end.

Miscellaneous. Nuts, bolts, screws, glue or double sided sticky tape.

You should be able to find these parts at your local R/C hobby store, or contact:

Tammies Hobbies    1 800-521-5614    12024 SW Canyon Rd. Beaverton OR  97005

Tower Hobbies         1 800-637-4989     PO Box 9078, Champaign, Illinois 61826-9078

Assembly Guide:

The Forearm:

The Forearm contains the forearm base, the grippers, and mini servo’s.  One servo controls the gripper assembly, and the second servo attaches to the shoulder assembly to move the arm up and down.

The finger blocks are cut into two equal pieces, and then mounted to the aileron bellcrank.  Two holes are drilled into the forearm base to mount the aileron bellcranks.  The bellcranks are then trimmed so they don’t interfere with each other.  Foam rubber or rubber feet can be mounted to the fingers to give them a good grip.

Unravel the braided push rod cable and divide it to form a ‘Y’ joint.  Soldered the cable at the split of the ‘Y’ joint.  Three mini snaps are glued with epoxy to the push rod ends.  It is important that you measure each length in the ‘Y’ carefully because it needs to fit exactly between the bellcranks, and the servo control horn.

Approx. 5 1/4 inches.

The push rod cable travels through a small piece of push rod tube, and attaches to the gripper servo with a mini snap.  The push rod is then held down with a gear clamp.

Note:  Several components of the ARMBOT can be attached together with 3M double sided foam tape.  This works well with the shoulder hinge attaching to the shoulder block and the forearm base.  The tape also works with shoulder servo.  Using the 3M tape decreases the amount of time it takes to build the ARMBOT, and you don’t have drill as many holes.

The Shoulder Assembly:

A 3/16 inch hole needs to be drilled through the center of the shoulder block.  The disc control horn is attached to the shoulder block bottom, and the plastic hinge is attached to the front of the shoulder block.

The 2-56 push rod is cut and bent to the shape of an ‘L’.  A mini snap connector is mounted to the threaded end of the ‘L’ rod. The mini snap connector will attach the shoulder to the shoulder block.  This push rod will lower and raise the forearm.

The other end of the ‘L’ shaped push rod is inserted into s small piece of the push rod tube.  The tube is mounted to the shoulder assembly with one gear clamp.  Refer to the diagram.

ARMBOT Side View:

The Base:

The base is used to hold the rotation servo and the BOTBoard microcontroller.  Use foam tape to mount the shoulder rotation servo in place. The base has four rubber feet mounted to the bottom.

The BOTBoard is located on the opposite end from the rotation servo and is mounted with standoffs.

The three servos of the ARMBOT are connected to three of the servo ports on the BOTBoard.  Refer to the BOTBoard manual on programming the servos.

Software can be written for the BOTBoard to allow the positioning of the ARMBOT from any computers serial port.  In this was you could program the ARMBOT to move with simple BASIC commands from your PC.  You can also program the BOTBoard to act as a simple sequencer to move the ARMBOT around.  Sensors could give the ARMBOT a sense of touch, or allow it to see objects of light or dark.

This kit was designed to give you only the custom built parts.  That way I keep costs low and provide a service to help you engineer your own robot arm.

If you have any question, or suggestions for improvement, please contact me at:

Marvin Green

821 SW 14th

Troutdale, OR  97060

(503) 666-5907

marvin@agora.rain.com